1. INTRODUCTION

1.1 Background

Machine-vision applications such as intelligent transportation, surveillance, industrial monitoring, and Internet of Things devices require image sensors that remain active for long periods. Continuous capture and processing using conventional architectures consume considerable energy because raw images must be converted into digital form and transferred from the sensor to a separate processor.

PWM CMOS image sensors represent image intensity through pulse duration. Compared with conventional voltage-readout sensors, this approach can provide low-voltage operation, low power consumption, and a wider dynamic range. Nevertheless, transmitting every captured image to an external processor continues to waste energy, especially when a scene remains static.

Processing-in-sensor places selected computational functions close to or inside the image sensor. This reduces data movement by extracting relevant information before it leaves the sensor. The base paper extends this approach by integrating motion detection and CNN-based feature extraction into a reusable in-column mixed-signal computing core.

1.2 Problem Statement

Existing always-on vision systems face three major limitations:

- Redundant processing: Static or irrelevant frames are continuously transmitted and processed.

- Reduced pixel fill factor: In-pixel frame memory and motion-detection circuitry increase pixel complexity and reduce the light-sensitive area.

- High peripheral power: Comparators, ADCs, memories, and external processors contribute significantly to total energy consumption.

A low-power architecture is therefore required to detect motion continuously and activate more expensive feature extraction only when useful activity occurs.

1.3 Objectives

The objectives of the project are:

- To study an in-column mixed-signal computing core for PWM image sensors.

- To understand the cooperative operation of motion detection and feature extraction.

- To examine frame-difference motion detection without additional in-pixel memory.

- To analyse PWM-based weighted current integration for CNN computation.

- To study the implementation of convolution, ReLU, and max-pooling operations.

- To evaluate the architecture using power, accuracy, area, and energy-efficiency results.

- To identify limitations and possible improvements for future always-on vision systems.

1.4 Scope

The work focuses on the architectural and functional analysis of the computing core presented in the base paper. It covers its PWM interface, motion-detection process, feature-extraction process, dynamic comparator, control flow, prototype measurements, and MNIST classification results.

2. LITERATURE REVIEW

PWM CMOS image sensors convert photocurrent-dependent information into time-domain pulse signals. Earlier PWM sensor designs demonstrated low-voltage operation, wide dynamic range, and reduced energy per frame. However, they primarily focused on efficient image capture rather than integrated intelligent processing.

Processing-in-sensor architectures subsequently introduced operations such as edge detection, convolution, feature extraction, region-of-interest detection, and classification near the pixel array. These techniques reduce off-chip data traffic, but continuous feature extraction remains inefficient when the observed scene is static.

Previous motion-detection sensors commonly used analog storage capacitors and processing elements inside each pixel. Although such designs enable local frame differencing, they increase pixel area and reduce fill factor. Other designs combine motion detection with region-of-interest extraction or handcrafted feature algorithms. These methods can reduce data volume but may not retain complete contextual information or support programmable CNN operations.

In-column processing offers a useful compromise. It preserves a relatively simple pixel architecture while allowing shared computation across columns. The base paper builds on this concept by combining frame-difference motion detection and programmable CNN feature extraction within the same mixed-signal hardware.

3. EXISTING TECHNIQUES AND PROPOSED SOLUTION

3.1 Conventional Sensor-Processor Architecture

A conventional vision system contains a pixel array, readout circuit, ADC, frame memory, communication interface, and external image processor. Every frame is digitized and transmitted even when the scene has not changed.

Advantages

- Supports general image-processing algorithms.

- Offers high numerical precision.

- Uses mature digital processing tools.

- Allows flexible software updates.

Disadvantages

- High ADC and data-transfer power.

- Large memory bandwidth.

- Continuous processing of static frames.

- Unsuitable for tightly constrained always-on applications.

3.2 In-Pixel Motion Detection

In-pixel techniques store information from consecutive frames and compute their difference locally.

Advantages

- Motion is detected close to the sensing element.

- Raw-frame transmission can be reduced.

- Rapid local response is possible.

Disadvantages

- Additional transistors and capacitors enlarge each pixel.

- Pixel fill factor and imaging quality may decrease.

- Scaling to high-resolution arrays becomes difficult.

- Device mismatch and leakage can affect accuracy.

3.3 Region-of-Interest and Traditional Feature Extraction

Some sensors combine motion sensing with ROI extraction, segmentation, local binary patterns, or histogram-of-oriented-gradient features.

Advantages

- Reduces the amount of transmitted image data.

- Allows object-dependent processing.

- Can operate at relatively low power for specific tasks.

Disadvantages

- ROI output may omit global contextual information.

- Handcrafted features provide less adaptability than learned CNN features.

- SRAM, ADCs, and digital logic may still consume considerable power.

- Separate functional modules introduce area and control overhead.

3.4 Proposed Technique

The proposed technique uses a general-purpose in-column computing core connected directly to PWM pixel outputs. The same core supports two operating modes:

- Motion-detection mode: Constant-current integration and capacitor-based subtraction perform frame differencing.

- Feature-extraction mode: Programmable weighted-current integration performs CNN multiply-accumulate operations, followed by ReLU and max-pooling.

The motion detector acts as a wake-up mechanism. The system remains in low-power MD mode during static periods and activates FE only when the accumulated number of changed pixel blocks exceeds a configured threshold.

4. SYSTEM DESIGN AND METHODOLOGY

4.1 Overall Architecture

The computing core consists of:

- Four mixed-signal computing cells.

- A finite-state-machine-based digital controller.

- A weight-loading module.

- Current-steering digital-to-analog converters.

- A decision-making module.

- PWM control logic.

- A dynamic tail-current comparator.

- Output logic for motion and feature signals.

The controller switches between MD and FE modes at frame boundaries. When no motion is detected, low-power MD continues. When motion is detected, the system executes a mandatory five-frame feature-extraction sequence before returning to MD mode.

4.2 Operational Flow

text

PWM image acquisition        |        vLow-power frame-difference motion detection        |        vDoes the event count exceed the total threshold?        |    +---+---+    |       |   No      Yes    |       |Continue   Activate feature extractionMD mode    at the next frame boundary            |            v   3 x 3 convolution            |            v           ReLU            |            v      2 x 2 max-pooling            |            v   Output feature information            |            v   Return to MD after the   prescribed FE sequence

4.3 PWM-Based Current Integration

The image intensity is represented by the width of a PWM pulse. The pulse controls how long a current is integrated on a capacitor. In feature-extraction mode, the current magnitude represents the kernel weight. Consequently, the accumulated charge is proportional to the product of image input and weight.

The fundamental relationship is:

Q=IT=CV

where:

- $Q$ is the accumulated charge,

- $I$ is the integration current,

- $T$ is the pulse duration,

- $C$ is the capacitor value, and

- $V$ is the resulting capacitor voltage.

The paper represents the signed accumulation as:

Qs=XiIxTx=CPVP+CNVN

Positive and negative current paths allow signed-weight computation. The hardware weight range is -8 to +8.

4.4 Computing Cell

Each computing cell contains four capacitors, current-mirror devices, switches, and a comparator. The capacitors store two sets of information and enable subtraction by switching their plates. This makes the same cell suitable for both frame-difference motion detection and convolutional accumulation.

A reference voltage of 600 mV is used to balance computational accuracy and energy efficiency. Circuit reuse reduces the area overhead that would otherwise result from separate MD and FE modules.

4.5 Motion Detection

Motion detection is performed by comparing information from the current and previous frames. Pixel groups are sampled, and constant currents are integrated onto separate capacitors during even and odd frames. Capacitor plate switching produces the frame-difference voltage:

VFD=VCR-VPR+δ

where:

- $V_{CR}$ is the current-frame accumulated voltage,

- $V_{PR}$ is the previous-frame accumulated voltage, and

- $\delta$ represents error caused mainly by charge leakage.

Two thresholds, $V_H$ and $V_L$, are placed symmetrically around half the reference voltage. A pixel block is marked as changed when its difference exceeds the upper threshold or falls below the lower threshold.

For the reported experiment:

- $V_H = 320$ mV

- $V_L = 290$ mV

- MD frame rate = 250 fps

- Analysed central region = 63 x 63 pixels

The changed-block outputs are accumulated. If the total exceeds $V_{TH}$, motion is declared and FE begins at the next frame boundary.

4.6 Feature Extraction

Feature extraction uses a 3 x 3 convolution kernel with a stride of three. Nine programmable weighted currents perform multiply-accumulate operations on the PWM inputs.

The sequence is:

- Accumulate weighted inputs from three even rows.

- Accumulate weighted inputs from three odd rows.

- Compare each result with half the reference voltage.

- Reset negative results to zero to implement ReLU.

- Compare the even-row and odd-row results to perform 2 x 1 max-pooling.

- Combine two adjacent column outputs through an OR operation to complete 2 x 2 max-pooling.

- Convert the selected analog value back into a PWM output using a ramp comparison.

A working cycle performs two 3 x 3 convolutions and one 2 x 2 max-pooling operation in 24 microseconds.

4.7 Dynamic Tail-Current Comparator

The comparator contains a two-stage dynamic amplifier and a positive-feedback loop. Its operation alternates between two phases:

- Active phase: A charged capacitor supplies a significant tail current, providing sufficient gain and decision speed.

- Standby phase: The direct current path is removed, sharply reducing average power while the load capacitance retains the output state.

The comparator operates with a 100 MHz pulse control signal and consumes an average of 0.43 microwatts.

4.8 Experimental Method

The experimental setup uses an FPGA to produce PWM signals corresponding to 8-bit grayscale images. These signals are sent to the fabricated chip. The output PWM signals are returned to the FPGA, counted, and converted into digital values.

For motion detection, the FPGA sums changed-block outputs and makes the final threshold decision. For feature extraction, it transfers the converted feature values to a computer or subsequent CNN layers.

The prototype was fabricated using 0.18-micrometer 1P6M CMOS technology and operates from a 0.8 V supply.

5. RESULTS AND COMPARATIVE ANALYSIS

5.1 Prototype Specifications

| Parameter | Value |
| --- | --- |
| CMOS technology | 0.18 micrometer 1P6M |
| Supply voltage | 0.8 V |
| Computing-core area | 720 x 272 micrometers |
| Number of computing cells | 4 |
| Input and output representation | PWM, quantizable to 8 bits |
| Weight range | -8 to +8 |
| Test frame rate | 250 fps |
| I MD effective period | 8.064 ms |
| FE working cycle | 24 microseconds |
| MD average core power | 5.23 microwatts |
| FE average core power | 17.53 microwatts |
| Computing-cell FE efficiency | 1.5 TOPS/W |
| Comparator power | 0.43 microwatts |

5.2 Motion-Detection Results

The motion detector was evaluated using images containing a moving square. The size, direction, and displacement of the square were varied.

The main observations were:

- The number of detected events generally increased with object displacement and speed.

- Larger objects produced more detected events.

- Very slow or very small objects were more difficult to detect at high frame rates.

- Reducing the frame rate increased inter-frame displacement and improved slow-motion sensitivity, but also increased system response time.

- Objects leaving the image boundary could produce slower event growth or fewer events.

These results demonstrate the expected trade-off between motion sensitivity, frame rate, and response latency.

5.3 Multiply-Accumulate Accuracy

The output PWM width showed proportional behaviour with changes in both input value and positive weight. Negative results were suppressed by the ReLU operation.

The measured MAC root-mean-square error was 2.2 digital levels, corresponding to 1.2% of the normalized full-scale range. This indicates that the analog computing path maintains useful linearity and precision for low-complexity feature extraction.

5.4 Feature-Extraction Demonstration

A 120 x 120 input image was processed using edge-extraction kernels and max-pooling. The resulting PWM data were converted into a 20 x 20 grayscale feature map. The reduced feature map preserved sufficient object information for rough visual recognition while substantially reducing the volume of data passed to subsequent processing stages.

5.5 MNIST Classification

The test network contained two convolutional layers, two max-pooling layers, and two fully connected layers. The proposed hardware computed the first convolution and pooling stages using eight kernels.

| Implementation | Classification accuracy |
| --- | --- |
| Floating-point software weights | 95.99% |
| Quantized software weights from -8 to +8 | 95.95% |
| Proposed hardware computing core | 95.09% |

The hardware result is 0.86 percentage points below the floating-point software baseline. Most of the degradation is therefore attributable to analog nonidealities rather than weight quantization alone, although the total reduction remains modest.

5.6 Power Analysis

MD consumes 5.23 microwatts, whereas FE consumes 17.53 microwatts. Thus, the low-power MD state uses approximately 3.35 times less power than FE.

For an always-on system, average computing-core power depends on the fraction of time spent in FE mode. Ignoring mode-transition overhead, it can be expressed as:

Pavg=1-dPMD+dPFE

where $d$ is the FE duty cycle. Using the measured values:

Pavg=1-d5.23+d17.53 microwatts

For example, at a 10% FE duty cycle:

Pavg=6.46 microwatts

This demonstrates why event-triggered FE is more efficient than continuous FE.

For the proposed 120 x 120 sensor-level configuration, the paper estimates total power of 51.7 microwatts in MD mode and 87.6 microwatts in FE mode, including the pixel array and multiple computing cores.

5.8 Advantages

- Low power: The system remains in MD mode during static periods.

- Reduced data movement: Feature information is produced within the sensor.

- Preserved fill factor: The computational circuits are placed in the column architecture.

- Circuit reuse: The same capacitors and comparator support MD and FE.

- Programmability: Signed weights support different feature kernels.

- Scalability: Multiple cores can be deployed according to array dimensions.

- Good accuracy: Hardware MNIST accuracy remains close to the software result.

- High speed: MD and FE were demonstrated at 250 fps.

5.9 Limitations

- Detection performance decreases for very slow or very small objects at high frame rates.

- Charge leakage introduces frame-difference error.

- Analog computation is affected by device mismatch, process variation, temperature, and noise.

- Thresholds require optimization for a specific scene or application.

- The hardware demonstration covers only early CNN layers and the relatively simple MNIST dataset.

- FE is forced to operate for five frames after a motion event, which may waste energy when the event is brief.

- The reported full-system power values are partly extrapolated rather than measured on a completely integrated sensor.

6. CONCLUSION AND FUTURE SCOPE

6.1 Conclusion

The base paper presents a mixed-signal computing core for always-on PWM CMOS image sensors. Its principal contribution is the cooperation between low-power motion detection and CNN-based feature extraction. Frame differencing monitors the scene, while feature extraction is activated only after motion is detected.

By processing PWM signals directly through current integration, the design avoids continuous high-power conversion and raw-image transfer. In-column placement prevents the loss of pixel fill factor associated with in-pixel processing. Hardware reuse allows capacitors, integration paths, and the comparator to support both motion detection and feature extraction.

The fabricated 0.18-micrometer CMOS prototype operates at 0.8 V and consumes 5.23 microwatts in MD mode and 17.53 microwatts in FE mode at 250 fps. It achieves 1.5 TOPS/W at the computing-cell level and a normalized MAC error of 1.2% of full scale. Its MNIST accuracy of 95.09% is only 0.86 percentage points below the floating-point software baseline. These results show that cooperative motion-triggered processing is a practical approach for energy-constrained, always-on vision applications.

6.2 Future Scope

Future developments may include:

- Integrating the pixel array and all computing cores on a single chip for direct system-level measurement.

- Introducing adaptive or learned motion thresholds to improve robustness under changing illumination.

- Compensating for capacitor leakage, mismatch, and temperature variation.

- Dynamically adjusting frame rate according to observed motion.

- Using multiple or irregular detection windows to improve spatial coverage.

- Expanding programmable weight precision and supporting additional CNN layers.

- Evaluating the architecture on more complex datasets and real-world object-detection tasks.

- Using event duration to select the number of FE frames instead of enforcing a fixed five-frame sequence.

- Adopting smaller CMOS technology nodes or 3-D integration to reduce area and energy.

- REFERENCES

- [1] J. Gao, Y. Zhao, Z. Wang, L. Chen, A. Zhan, Q. Jiang, F. Zhang, Y. Chen, and S. Qiao, "A General-Purpose Computing Core With Cooperative Motion Detection and Feature Extraction for Always-On PWM Image Sensors," IEEE Transactions on Very Large Scale Integration Systems, vol. 33, no. 10, pp. 2728-2739, Oct. 2025, doi: 10.1109/TVLSI.2025.3580675.

- [2] S. Hanson, Z. Foo, D. Blaauw, and D. Sylvester, "A 0.5 V Sub-Microwatt CMOS Image Sensor With Pulse-Width Modulation Read-Out," IEEE Journal of Solid-State Circuits, vol. 45, no. 4, pp. 759-767, Apr. 2010.

- [3] A. Y.-C. Chiou and C.-C. Hsieh, "An ULV PWM CMOS Imager With Adaptive-Multiple-Sampling Linear Response, HDR Imaging, and Energy Harvesting," IEEE Journal of Solid-State Circuits, vol. 54, no. 1, pp. 298-306, Jan. 2019.

- [4] T.-H. Hsu et al., "A 0.8 V Intelligent Vision Sensor With Tiny Convolutional Neural Network and Programmable Weights Using Mixed-Mode Processing-in-Sensor Technique for Image Classification," IEEE Journal of Solid-State Circuits, vol. 58, no. 11, pp. 3266-3274, Nov. 2023.

- [5] H. Xu et al., "Senputing: An Ultra-Low-Power Always-On Vision Perception Chip Featuring the Deep Fusion of Sensing and Computing," IEEE Transactions on Circuits and Systems I: Regular Papers, vol. 69, no. 1, pp. 232-243, Jan. 2022.

- [6] X. Zhong, M.-K. Law, C.-Y. Tsui, and A. Bermak, "A Fully Dynamic Multi-Mode CMOS Vision Sensor With Mixed-Signal Cooperative Motion Sensing and Object Segmentation for Adaptive Edge Computing," IEEE Journal of Solid-State Circuits, vol. 55, no. 6, pp. 1684-1697, Jun. 2020.

- [7] T.-H. Hsu et al., "A 0.5-V Real-Time Computational CMOS Image Sensor With Programmable Kernel for Feature Extraction," IEEE Journal of Solid-State Circuits, vol. 56, no. 5, pp. 1588-1596, May 2021.

- [8] I. Park, W. Jo, C. Park, B. Park, J. Cheon, and Y. Chae, "A 640 x 640 Fully Dynamic CMOS Image Sensor for Always-On Operation," IEEE Journal of Solid-State Circuits, vol. 55, no. 4, pp. 898-907, Apr. 2020.

- [9] K. D. Choo et al., "Energy-Efficient Motion-Triggered IoT CMOS Image Sensor With Capacitor Array-Assisted Charge-Injection SAR ADC," IEEE Journal of Solid-State Circuits, vol. 54, no. 11, pp. 2921-2931, Nov. 2019.

- [10] M. Lefebvre, L. Moreau, R. Dekimpe, and D. Bol, "A Programmable Convolutional Imager SoC With In-Sensor Current-Domain Ternary-Weighted MAC Operations for Feature Extraction and Region-of-Interest Detection," in IEEE International Solid-State Circuits Conference, pp. 118-120, Feb. 2021.